1.0 INTRODUCTION: There are two distinct methods of providing personal protection against contaminated atmospheres. They are:

- BY PURIFYING THE AIR BREATHED

The inhaled air is drawn through a medium that removed the contaminated air, the chemical & physical nature of the medium depending on the contaminating agent. Devices to achieve this result are known as respirators. The different types of respirators are represented below:

RESPIRATORS

High Efficiency Dust

Cartridge Type

FOR DUSTS

FOR GASES

FOR GASES WITH DUSTS

General Purpose

Dust

Positive

Pressure

Powered

Respirator

Hood or

Blouse

Canister

Type

Canister

Type with

Particulate

Filter

Cartridge

Type

Emergency

Escape

- BY SUPPLYING AIR OXYGEN FROM AN UNCONTAMINATED SOURCE

The inhaled air is conveyed by air line, or alternatively, air or oxygen is supplied from cylinders carried by the person exposed to the hazard. Devices that achieve this are known as breathing apparatus. The main types of breathing apparatus are represented below:

BREATHING APPARATUS

SELF CONTAINED

AIR LINE

CLOSED

CIRCUIT

OPEN CIRCUIT

EMERGENCY

ESCAPE

FRESH

AIR

COMPRESSED

AIR

2.0 RELATED FACTORS (when making a selection of RPE):

The selection of Respiratory Protective Equipment (RPE) must also take into account:

- the person chosen to carry out the tasks,

- the demands of the job to be undertaken

- the working environment

These points must always be considered as they may influence effective protection.

2.1 Personal Related Factors

These would include training, medical fitness, face shape, size and beard. Also whether person wears spectacles or contact lenses.

2.2 Work Related Factors

These would include physical work rate, length of time RPE is to be worn, mobility, visibility and effect on communication.

2.3 Working Environment Factors

Selection considerations on this are shown on the diagram overleaf.

2.3 WORKING ENVIRONMENT FACTORS

These indicate various air-borne contamination hazards under different environmental conditions.

NORMAL WORKING CONDITIONS

CONFINED SPACES

ALLOWING UNRESTRICTED MOVEMENT

ALLOWING LIMITED MOVEMENT

ALLOWING UNRESTRICTED MOVEMENT

DUST HAZARDS

GAS HAZARDS

COMBINED DUST & GAS

DUST

GAS

LOW O2

DUST

GAS

LOW O2

ALL HAZARDS

ALL HAZARDS

AIR LINE

BREATHING APPARATUS

SELF CONTAINED BREATHING APPARATUS

(SCBA)

TOXIC

LOW TOXICITY

TOXIC

LOW TOXICITY

TOXIC

CARTRIDGE

CANISTER

or SCBA

CANISTER or SCBA

Positive Pressure Respirator

Positive Pressure Hood & Blouse

General Purpose Respirator

Positive Pressure Hood & Blouse

High Efficiency Dust Respirator

Self Contained Breathing Apparatus (SCBA)

3.0 EXAMPLE OF SELECTION OF RESPIRATORY PROTECTION EQUIPMENT

- Identify the name, nature and form of  air borne contaminant.

- Establish hazard and harmful effects of contaminant.

- What legislation, regulations etc., are applicable to the substance?

- What Occupational Exposure Limits (OEL) are set for the contaminant? (see Code of Practice for  Safety, Health & Welfare at Work (Chemical Agents) Regulations, 1997 )

- Obtain airborne concentration of substance in work area  (milligrams per cubic metre or ppm).

- Length of exposure TWA (Time Weighted Average) to the one substance.  (see note 3 below).

- Divide TWA airborne concentration by the OEL for that substance to ascertain level of protection required

- Select the respirator which has a Nominal Protection Factor greater than the required level of protection. (See notes 1 and 2 below on protection factors).

Example:

Nature of contaminant:					Graphite

Form of contaminant:					Dust

Health effects:						Respiratory sensitivity

Concentration:						50 mg/m3

OEL. for Graphite :	 				  5 mg/m3

Total exposure time:					  6 hours

8 hour Time Weighted Average concentration:		50 mg/m3 x 6 = 37.5 mg/m3

8

Divide TWA concentration by OEL	    		37.5 mg/m3  = 7.5

5  mg/m3

Result:    7.5 x OEL.

Therefore select a general purpose respirator that has a Nominal Protection Factor greater than 7.5. (see notes 1 and 2).

Note.1  Nominal Protection Factor (N.P.F.) is the ratio of the concentration of the contaminant in the atmosphere to its concentration in the air inhaled by the wearer of a respiratory protection device viz.

Concentration of contaminant in the atmosphere   	= 	N.P.F.

Concentration of contaminant in the facepiece

This is calculated at a maximum inward leakage in prescribed tests.

Note.2  While the above example is a simple selection method, one must also take into consideration the various factors previously outlined in 2.0 Related Factors.  However, in the revised BS4275 ‘Guide to implementing the effective respiratory protective device programme’, the use of Nominal Protection Factors are replaced by Assigned Protection Factors (APF’s).  The difference that APF’s have made is that RPE should be selected with a higher margin of safety.  Advice should be sought from suppliers of RPE regarding selection, as for example a full face piece respirator with a P3 filter formally had an NPF of 10 whereas its APF was found to be 40.

Note.3 Occupational Exposure Limits (OEL’s) are only for one substance in the atmosphere.  In the case of multiple substances being present in an atmosphere, special attention should be given to ‘synergistic effects’ and appropriate downward revisions of an OEL for an individual substance.